AN - 2 TP SP-OKO (SAFARI) Basic aircraft data Type / model AN-2 TP / SP-OKO (safari) Year manufactured 1973 Manufacturer PZL Mielec, Polska Factory serial number 1 G 13901 Register number 3937 Certificate of Airworthiness valid through August 26, 2009 PZU SA TPL and Aerocasco comp. insurance valid through June 2, 2009 Total flight time since last overhaul / Last overhaul date - airframe 1398 hours 49 min / August 13, 1986 - engine 212 hours 19 min / March 31, 2005 - propeller 203 hours 49 min / April 15, 2003 Total flight time since beginning of operation 3417 hours 50 min. Operating time to overhaul 1101 hours 11 min.

Operating data Crew 2 Number of passenger seats 6 Cargo area up to 200 kg

Technical data Dimensions wingspan: 18.18 m length: 12.40m height: 4.10 m lift area: 71.50 m2 Weight empty weight: 3450 kg usable payload: 2050 kg takeoff weight: 5500 kg fuel tanks: 1200 l fuel consumption: approx. 160 l/h Specifications maximum speed: 258 km/h cruising speed: 185 km/h minimum speed: 90 km/h climbing speed: 3.5 m/s maximum altitude: 4400 m range: 900 km takeoff roll: 170 m landing roll: 215 m

Basic engine specifications Engine type, series ASz-62 IR, series 16 Year manufactured 1987 (engine installed an aircraft) Factory zero number K 16467193 I main overhaul WSK PZL Kalisz March 31, 2005 Total flight time since last overhaul / Last overhaul date - engine 212 hours 19 min / March 31, 2005 Time to Total overhaul 787 hours 41min of operation

Basic propeller specifications Propeller type AW-2 Manufacturing date October 26, 1988 (propeller installed an aircraft) Factory zero number H 108430106 I main overhaul April 15, 2003 usage after overhaul 1500 hours or 6 years Total flight time since last overhaul / Last overhaul date: - propeller 208 hours 49 min / April 15, 2003 Calendar usage valid through April 15, 2009 Remaining to main overhaul 1291 hours 11 min. of operation

Radio-navigation equipment (installed an aircraft) Transceiver IC-A 200 ICOM Transceiver SL-30 „Garmin” No. 25902126 GI 106A „Garmin” Cross indicator No. B 06-10028 Transponder KT 76C Bendix / King ALTITUDE DIGITIZER A-30 „ACK” GPSMAP 296 „Garmin” No. 67104244

Additional, nonstandard equipment: Air-conditioning with remote control Sony audio system Safe Noise muffler Additional baggage container under the aircraft Package container for sun marquees External power from Honda generator (for use when parked) Drinkable and usable water tanks built into the aircraft structure Anti-sun awning for installation when parked, as well as a table and four folding easy chairs (camping version)

Description Lounge version of biplane, in very good technical and overall condition of passenger section equipment. It has a lounge with armchairs and couches made of yellow leather, a wooden table (dark brown), kitchen area with a refrigerator and microwave oven, kitchen cabinets hidden behind aluminum colored louvered doors, bathroom with shower, wash basin and WC. In the cockpit: yellow leather seats. Carpeting - gray. The entirety constitutes a very attractive and comfortable interior. Additional equipment: air conditioning and noise muffler increases comfort during flight. The presence of an electrical generator and tanks for usable and drinkable water, an anti-sun awning and a camping set allow for recreation and independence while standing on the ground, at any time of day.

Location: Poland

Contact Bernard Kwapioski telephone: +48 609 026 370 e-mail: bk@ekopark.pl